9200HD (PVR) b-07 LED Display Fix

This problem can occur by many reasons using the wrong bin file or a defective memory stick, etc.  You can fix the problem yourself as follows:

1. Click ‘9200HD’ in the download section and save the ‘9200HD (PVR).zip’ in your computer
2. Extract (unzip) the 9200HD (PVR).zip and then copy and paste the “API_FILE.bin” to the memory stick. (Note : The memory stick should not contain 9000HD API file)
Note : The ‘API_FILE.bin’ is a renamed file of ‘9200HD_100818_1229_api_pvr.bin’
           for repair purposes.

3. Insert the memory stick to the receiver.
4. Press and hold the volume up (right hand side) button gently (not too hard) and then turn the reset power button off and on.
5. The “API” will appear on the front LED.  After this you can release the volume button and wait about 40 seconds until “END” appears after following the display “BURN”, “SUCC”.
6. Reset the unit to reboot and display “ON” or any channel number.
Note : If it the unit still displays “INIT” or “----“ and does not show anything
           on the TV screen, reset the main power 2~3 times consecutively or while
           holding volume button and main power off and on until LED display
           mode red dot move left to right one by one and finally display “ON”.
7. If the “API” does not appear at item 4, try the above procedure again.
8. If it can not still be fixed, return the unit to the Panarex service department.
