SATELLITE POSSITIONER PROGRAMMING THE DRAKE APS 3240E

1/ Press ADD/ERASE and POWER at the same time for 1 second to enter the program mode

2/  PRESS east or west until dish  finds a feed

3/   PRESS ADD/ERASE to store satellite preset.  The lowest available memory location number will be assigned 

Then press power to exit the program OR keep putting in positions for other satellites

Up to 36 positions are available.  For normal operating mode press east or west buttons to display the satellite preset after 3 seconds the dish will go to that preset position.

Manually moving to other satellites.

To enter this mode.

Press east and west buttons at the same time the display will blank momentarily and then indicate the current log position ie the log light will be on then you can move to the east or west manually

                              PROGRAMMING THE DRAKE APS 3240E

==============================================================.

                                      To erase all the programming 

                                     =======================

Fist enter the program mode by simultaneously pressing add/erase and power buttons,

Next press the add/erase and limit/recall buttons at the same time for 5 seconds until the display reverts to the log position 98.

Please use extreme caution when performing this function since all memory contents has been erase

The limits are set to a fixed set memory contents are all erased.

And the position count is set to the middle of its range

                                PROGRAMMING THE EAST/WEST LIMITS

                              ====================================

1/  press limit/recall and power simultaneously for 1 second to enter the limits setting mode.

2/ press east until dish reaches desired limit

3/  press limit/recall 

4/ press east to  store

5/  press west until limit required.

6/  press limit/recall 

7/  press west to store   then press power to xit limit settings mode  

